SILICON CONTROLLED RECTIFIER

A-002-05-01

What are the three terminals of a silicon controlled rectifier (SCR)?

1 Gate, base 1 and base 2

2 Base, collector and emitter

3 Anode, cathode and gate

4 Gate, source and sink

A-002-05-02

What are the two stable operating conditions of a silicon controlled rectifier (SCR)?

1 Forward conducting and reverse conducting

2 Conducting and non- conducting

3 NPN conduction and PNP conduction

4 Oscillating and quiescent

A-002-05-03

When a silicon controlled rectifier (SCR) is triggered, to what other

semiconductor diode are its electrical characteristics similar (as measured

between its cathode and anode)?

1 The junction diode

2 The PIN diode

3 The hot-carrier diode

4 The varactor diode

A-002-05-04

Under what operating condition does a silicon controlled rectifier (SCR) exhibit

electrical characteristics similar to a forward-biased silicon rectifier?

1 When it is gated "off"

2 When it is used as a detector

3 During a switching transition

4 When it is gated "on"

A-002-05-05

The silicon controlled rectifier (SCR) is what type of device?

1 PNPN

2 NPPN

3 PNNP

4 PPNN

A-002-05-06

The control element in the silicon controlled rectifier (SCR) is called the:

1 anode

2 cathode

3 emitter

4 gate

A-002-05-07

The silicon controlled rectifier (SCR) is a member of which family?

1 Phase locked loops

2 Varactors

3 Thyristors

4 Varistors

A-002-05-08

In amateur radio equipment, which is the major application for the silicon

controlled rectifier (SCR)?

1 Power supply overvoltage "crowbar" circuit

2 Class C amplifier circuit

3 Microphone preamplifier circuit

4 SWR detector circuit

A-002-05-09

Which of the following devices has anode, cathode, and gate?

1 The bipolar transistor

2 The silicon controlled rectifier (SCR)

3 The field effect transistor

4 The triode vacuum tube

A-002-05-10

When it is gated "on", the silicon controlled rectifier (SCR) exhibits

electrical characteristics similar to a:

1 reverse-biased silicon rectifier

2 forward-biased PIN diode

3 reverse-biased hot-carrier diode

4 forward-biased silicon rectifier

A-002-05-11

Which of the following is a PNPN device?

1 PIN diode

2 Hot carrier diode

3 Zener diode

4 Silicon controlled rectifier (SCR)